EYT250 touchpanel

TPC vers 1.0.8 (Touch Panel Configurator)

update of EYT250 operating system and program can be done directly with TPC

EYT 250 new service release 1.2.2

new password functionalities  (guest, user, admin incl. 6 passw.-levels)

updating of the novaNet driver

sub-applications numbers can be written down to AS

toggle function included (light switching, stores a.s.o.)

EYT250: what`s new?

EYT 250 new service release 1.2.2

- an alarm is indicated by a flashing red LED light.

- more quick access to the pictures, less flickering of the touch-screen

- with “dummy.xml” an application can be started without an AS-conn.

- The EYT250 can read the AS- time. The AS and synchronizes each 30 minutes the time of the EYT250

- Set and reset function of counter values

- Filter functions:

AS filter (device.txt): single AS selection 3,10,45

from.. to.. Selection 46-123

Application-filter

without standard application (name of application)

with standart application (houseadress of AS)

Historical data will come in a later version (vers.3) end of the year

Other languages will come also end of this year

EYT250: sockets

1.) 0386380 001 connecting cable 230V

2.) 0367862 002 connecting cable EYT250 -> nova AS

3.) xxxxx „crosslink“  cable EYT250 -> computer

4.) yyyy DB9 „zero modem“ cable  EYT250 -> computer

two female plugs, wiring:

5  to  5

3  to  2

2  to  3

Roadmap EYT250
As of January 2004

typical application............

EYT250-touchpanel

screen overview novaclim               screen plate-heat-xchanger

screen cooling coil

screen heating coil

screen fans

screen setpoint

screen summer-comp. 		     screen FAK

overview parameters

parameters heating coil

control parameters                           service address

service tel.-no

State of the Market-Introduction novaFlex

- Flyer:
      in English, French and German available

- Demo-Case (available now!):
novaFlex and EYT250

- NSO prepares  some basic models for heating

- NSO prepares some basic models for air-conditioning

Fixed NSO applications

What do you need:

- Process-plan

- Tested project data of the application created in CASE Prj

- Tested project data of the application created in CASE FBD

- Wiring-diagram

- Function description

- Pictures and texts in original language for EYT250

- Structure of the pictures (operating concept)